SmartHub Windows App Requirements

Revised from the photographed pages so the document focuses on the actual SmartHub desktop system.

Scope note: SmartHub is a Windows desktop diagnostic tool for Android phones. It uses a WPF shell with WebView2, a local HTML/JavaScript UI, and a local Node/Express + TypeScript companion service.

# 1. System Overview

Primary platform: Windows desktop app for technician use.

Purpose: diagnose Android phones through ADB, Fastboot, USB-only triage, and optional webcam-based screen analysis.

Operating mode: local-first / offline-capable after initial setup, with optional cloud sign-in and history sync.

Target outcome: provide clear diagnostic verdicts, simple fixes, and technician-friendly summaries without relying on dialer codes.

# 2. Windows App Requirements

Core runtime and environment requirements for the Windows desktop app.

| Category | Requirement | Notes |
| --- | --- | --- |
| Operating system | Windows 10 or later, 64-bit | The desktop shell is built for Windows only. |
| Desktop runtime | .NET 8 WPF application | The Windows shell project targets net8.0-windows and uses WPF. |
| Browser runtime | Microsoft WebView2 | Required to render the local HTML/JavaScript UI. |
| Companion service | Local Node/Express + TypeScript backend | Runs on the same PC and listens on localhost for diagnostics. |
| Development / admin access | Permission to install and run the app locally | Needed for setup, updates, and connecting to devices. |
| Disk space | Enough space for app files, logs, screenshots, and optional toolchains | Exact minimum is not fixed in the repo. |
| Input devices | Keyboard and mouse | Recommended for technician workflows and data entry. |

# 3. Android Device Requirements

Requirements for the connected Android phone and the technician workflow.

| Category | Requirement | Notes |
| --- | --- | --- |
| USB connection | Working USB cable and stable USB port | SmartHub relies on USB to communicate with the phone. |
| ADB access | USB debugging enabled for full ADB diagnostics | Some features require ADB access; USB-only triage still works when debugging is off. |
| ADB tools | Android platform-tools available to the Windows app | Used for device detection, diagnosis, and fixes. |
| Drivers | Proper Android USB drivers on the PC | Important for reliable detection and data transfer. |
| Fastboot | Fastboot availability when troubleshooting boot issues | Used for bricked or bootloader-level diagnostics when the device supports it. |
| Device state | Phone should be connected and powered on when normal diagnostics are needed | USB-only blue/blank-screen triage can still help when the display is unusable. |

# 4. Optional Diagnostic Requirements

These are only needed for specific SmartHub features, not for the base Windows shell.

| Feature area | Optional requirement | Purpose |
| --- | --- | --- |
| No-USB-debugging / BSOD analysis | Python 3.8+ with OpenCV and NumPy | Used by the camera-based screen analysis helper. |
| Webcam-assisted checks | Working webcam | Helps identify blue/blank screen conditions and visible UI content. |
| Offline AI helper | Local Python support | Generates conservative local conclusions without Internet. |
| Online sign-in / cloud history | Internet access | Needed for first sign-in and optional cloud syncing. |
| Supabase history sync | Configured Supabase project and credentials | Stores per-user diagnostic history when enabled. |

# 5. Functional Requirements of the Windows App

Run ADB-based diagnostics for connectivity, hardware, OS health, and app/security checks.

Show technician-friendly summaries instead of raw dumps where possible.

Support no-USB-debugging / USB-only triage for broken-screen or blue/blank-screen devices.

Provide optional webcam-based screen analysis and offline AI guidance.

Allow controlled fixes for supported Android-side issues such as Wi-Fi, Bluetooth, GPS/location, and mobile data.

Keep diagnostics local by default and avoid unsafe host-PC network changes in Android-only mode.

Store diagnostic history locally and optionally sync to cloud storage per signed-in user.

# 6. Important Constraints

The app is a Windows desktop companion for Android diagnostics, not a generic PC repair tool.

Host-PC network fixes such as Winsock reset and DNS flush are intentionally blocked in Android-only mode.

AI conclusions should be conservative: only say No problems detected when the scan is clean enough.

If signals are mixed or incomplete, the app should report possible issue or no single clear cause yet instead of guessing.

Unknown-source apps should not be treated as risky by default unless they show behavior that can harm the phone or make it unresponsive.

# 7. What Was Revised

Replaced the generic classroom-system wording with SmartHub Windows app requirements.

Aligned the document with the actual repo stack: WPF/WebView2, Node/Express, TypeScript, ADB/Fastboot, and optional Python analysis tools.

Added the Android device prerequisites and optional feature dependencies so the document is usable as a real app requirement sheet.

Source basis: SmartHub README, Windows shell project file, and the diagnostic feature set in the repository.